# Navigation and Tracking

A tracking system receives noisy measurements or data or observations y1(x1,v(n)) ,y2,yn = y^n and seeks to estimate a time varying value or parameter or state x1,x2,xn = x^n which is embedded or hidden in the data.  The problem at hand is statistical inversion of the system to estimate the state at each time step given the available data.

In SLAM measurements provide a way to estimate target and landmark positions within some reasonable limitations.  In INS the state model provided with motion estimates can track by deadreckoning for some period of time.  VIO is the combination of both where motion sensors inform prediction through deadreckoning while measurements provide estimates of errors in the gyro and acel to extend the duration of accurate tracking.

## Reference Frames, Rotations, and 3-D Kinematics

Right handed coordinate system.  Typically, x is direction of travel, y goes left wrt x, and z is down.  The origin is wrt to an arbitrary reference frame.  In imaging systems, z is typically oriented parallel to the optical axis, x is aligned with the horizontal sensor axis and y is positive downward along the vertical sensor axis, and the origin is the optical center.

sensor frame - the sensor units

body frame - the vehicle, axis rigidly attached.  If there is no slipage velocity points along one of the axis

inertial frame - so called fixed space nonaccelerating where newton's laws are valid, e.g relative to fixed stars , this is what imu measures with respect to.  Though the earth is rotating slowly 15 deg/hr, it can serve as one for gyros not sensitive enough.

navigation frame - where position is wanted. example earth rotating

Kinematic equations for motion can be formulated in either DCM or quaternion representations.  DCM is more straightforward, quaternion is numerically superior.

In what follows we use an inertial frame to simplify the presentation.  Development in other frames needs to be looked up in a reference paper.d

One thing to note is that in the literature there are inumerable notations, conventions, definitions etc.  Therefore, it's best to go through it yourself, pick your own style and then learn to convert between implementations.  For example the cross product matrix can be defined up to a sign which leads to all sorts of confusion.

### Cartesian

The radial position of an object relative to the origin is

where ei are the unit vectors parallel to the principle axes of the reference frame.  If the object is moving

Note that since the unit vectors do not change with time, the first terms in the product derivatives are zero.  For moving objects the velocity is the sum of of the individual scalar coordinate speeds which can result in simplified expressions.  This will not be the case in general.  Similarly for acceleration we have

### Spherical

The dimensionless unit vectors constituting the spherical basis are  pointing in the direction of increasing radius,  pointing in direction of increasing azimuth, and  pointing in direction of inreasing inclination.  Note that the inclination is conceived such that theta=zero corresponds to z=0, whereas in some texts its 90-this value.   Together they form an orthogonal basis for the point r = (x,y,z) tangent to a sphere of radius r.

The unit vectors are defined such that they represent the direction the position vector will move in if we increase the corresponding coordinate leaving all others fixed.  Thus we have the definitions

Cartesian to Spherical

From () the unit direction vector parallel to  and perpendicular to the tangent plane therein is

From the unit vector definition in ()-() and expressing in terms of the rectangular basis leads to

#### Example 2D Motion

For circular motion the radial distance is constant, hence

where the velocity is perpendicular to r at all times.  For general 3D motion, as in Figure ().  There are three orthogonal motion directions:The spin axis omegau, the tangential rotational component of the velocity vperp, and r.

The tangential velocity (which is perpendicular to both r and u) can be recovered from () by right crossing both sides with r

Now let r be resolved in a body frame and let R be the matrix which transforms into an inertial frame.  For any basis vector e in the body frame its velocity in the inertial frame is

v=de/dt=omega x e.  R is composed of three of these basis vectors  e1 e2 e3, so dR/dt = omega x R is

de1/dt = omega x e1, de2/dt = omega x e2.  To prove this consider

Spherical to Cartesian

In spherical coordinates the unit vectors are functions of time and therefore have non zero derivatives expressed in terms of either rectangular or spherical basis.  Thus by the chain rule, the velocity is

The acceleration is derived similarly as

## Rotational Kinematics

Rotation of the body frame relative to the world/reference frame must be tracked over time.  Therefore, we need a data structure to keep track of this orientation, a method to apply an orientation change, and a method to evolve it over time.  The attitude can be described by a sequence of Euler angles around fixed axes which are embedded in a rotation matrix.  Or by an axis and a rotation around it followed by a special sequence of multiplications to .  The differential equations governing the time evolution can be approximated in a Taylor series since adjustments between closely space intervals are mostly small.  The equation can then be solved using a suitable number of terms for the application.  In some circumstances closed form solutions can be found.

It's good to group all the information about orientation paradigms into a single place: notation, rotation types, derivatives, differentital equations and solution via integration.

### Euler Angles

roll , Pitch , yaw .  The sequence of rotations introduces a series of intermediate frames.  Convention is first yaw rotation around the initial z-axis which rotates the initial x and y axes.  Second the pitch rotation is performed about the common y axis between the first and second frames.  This rotates the x and z axes in this frame.  Third the roll rotation is performed about the common x-axis between the second and final frame.  This rotates the z-y axes of the final frame.  When attitude describes the orientation of a body wrt the local navigation frame, roll, pitch and yaw are known as bank, elevation, and heading or azimuth, respectively.

From the third column and first row, taking ratios of appropriate elements to eliminate angles yields the reverse transformation

Euler angles are discontinuous at their aliasing points therefore when manipulating these angles they must be wrapped back into the nominal range.  Also, averaging is possible only over intervals which do not contain a break.

From Groves

#### Derivatives

Derivatives of rotation matrices with respect to time can be computed via the definition of the derivative as in section 22.11.1.  When rotation rate is formulated in a moving body frame the differential rotation is the skew symmetric matrix we encountered in (linalg)

multiplies the world to body rotation matrix from the right (because the right side is still in body frame).  The Jacobian wrt to its argument is

For rotation rate in a stationary reference frame, the skew matrix multiplies from the left.  Thus

It's always important to keep in mind which frame multiplies are allowed.  Right multiplies are in body frame, and left wrt reference.

Alternatively,

which implies  is skew symmetric, say , and

.

Note the rotation matrix R(u,mu) and Omega(u) commute (proof via Rodirigues) if the axis is the same.  The final result of both operation sequences is the same since they share the same axis.  You can rotate x around u, the take the cross-product which will be perpendicular to u, or you can take the cross-product of x and u, then rotate that a smaller amount.  Ends up in the same place.

This differential equation can be solved by exponentiation to yield

with R(0) = I signifiying body aligned with reference at t=0.  Since R(t) and R(0) are rotation matrices, so is eOmega.  Mathematically, () exponentially maps the tangent space so(3) to the rotation space SO(3) or angular velocity to its corresponding infinitetesimal rotation matrix.   Here, omega is in the body frame which implies right multiplication.

Let  define an axis and  the associated rotation angle.  Then based on () we can define the rotation exponential and link it to Rodrigues's formula as

Consider the derivative of R with respect to scalar angle psi.

Therefore, the derivative wrt to yaw is the product of a skew symmetric matrix containing the yaw axis of rotation and the original rotation matrix

We can use () to compute rotation change with respect to attitude.

Integration (Note this section Omega is in world frame)

We can represent this change in angle in terms of a differential or infinestesimal rotation matrix which we now show.  Expanding the angle in  in a first order taylor series

And using sum trigonometric formulas and small angle approximations for sin and cos yields

Where the skew-symmetric matrix

Represents the infinitesimal pitch rate.  Similarly for roll and yaw/heading, we have

The matricesare indeed rotation matrices to first order (e.g. in the limit of dt going to zero),

And they commute,

These commuting infinitesimal rotation matrices can be used to differentially propagate a general rotation matrix  by a small amount as

Which we write as

Where

Is the negative crossproduct matrix often denoted  for the rate vector

The time derivative of the coordinate transformation matrix is defined as

Substituting  in  yields the differential equation for pose

Here, R is the camera to world matrix.  Hence Omega is in world since it left multiplies (already in world) When we wish to emphasize the dependence of  on the individual rates we write .

Assuming is constant over the update interval the solution to  is found using the matrix exponential in

Using

The closed form solution to () is

which is the unnormalized Rodrigues representation of a matrix.  The exact discretized pose propagation equation over sampling interval  for the rate vector  is

where we have substituted sincs to equalize powers.

### Angle-Axis

From Section Matrix Analysis

Example: rotate around z-axis

u  = 0 0 1, mu = pi/4

q = cos mu/2 u*sinmu/2 = (q0 v)

R = 2q0^2-1 * I + 2vv' + 2q0K =

R =

    0.7071    0.7071         0
   -0.7071    0.7071         0
         0         0    1.0000

#### Properties of Omega Matrix

### Direction Cosine Matrices

Elements are basis vector dot products (cosines) between two coordinate frames.

### Quaternions

Mathematically, quaternions generalize complex numbers.  Whereas multiplying by complex number  rotates a vector in the plane, the quaternion,

rotates it in 3D about an axis defined by .  In fact there is an entire language of quaternions comprised of identities, rules and derivatives.  Practically, quaternions have advantages compared to rotation matrices for keeping track of rotations.  Also there are different conventions and orderings.  We present the Hamiltonian style here.

The key result is the quaternion rotation operator

which will be developed after the following mathematically preliminaries.

While quaternions add in the normal way the rules of multiplication are special and are defined to ensure operations result in a non-commutative field.  As such quaternion algebra satisfies closure, associativity, distributivity, identity and inverse.  It was precisely the statistfaction of these properties that confounded the inventor W.R. Hamilton for over a decade in the mid 19th century.

#### Algebra

Multiplication is defined by the following special products:

From here one can show the product of two quaternions p and q,  is

It is now convenient to introduce the notation

where q0 and v are the scalar and vector part, respectively.  q0 = 0 corresonds to a so called pure quaternion.  The addition of a scalar and vector is indeed a strange proposition but as long as calculations are performed according to the rules everything works out in the end.  Using () in () and the definitions of cross and dot products results in a so called working definition of quaternion multiplication, namely

Often we omit the special multiplication system writing .

We can further abstract the notation to a pure 4-vector

,

in which case we write for (),

For a pure quaternion and  () reduces to

Expanding out the multiplication, the components of Eq () can also be written as left and right matrix multiplication via

Eq () can be written more compactly by identifying terms to express as

These matrices come up enough to warrant a special symbol,

where L and R indicate if either the left or right quaternion in the product is converted to matrix form.  Then

Even more, when using the left multiplication matrix it can be simplified from 4x4 to 4x3 by dropping the first column since omega is a pure quaternion, in which case it is typically denoted as

It will also be convenient to define the right multiplication matrix of a pure quaternion as

which is turns out to describe how a quaternion changes in response to an angular velocity.

If u=v, then pq = uv = |u|2 + 0, and if u is one, then = -1.  Based on this fact we can expand compute powers of v = umu as

From here we can define the exponential of a pure quaternion as

For small angles, and up to first order () is approximated as

The identity quaternion represents no rotation and is defined as (1 0 0 0) becuase

As a result q*a+q) = q*(a+1)

Conjugation is governed by

and

which can be proved by using the definition of conjugation in ().

The norm is defined by

The inverse is

#### Rotation Operator

Note, that if q is unit length, the inverse is equal to the conjugate and q is unitary.  From now on we assume unit quaternions.  Indeed all quaternions describing rotations have this property.  For such unit norm quaternions , hence we can define

It will be shown next that angle theta is related to rotation around the unit axis u.  To get a feel for this consider the product

The product of two same axis quaternions acts identically to the product of two rotation matrics.  Changing to the conjugate rotates in the opposite direction.  Now consider the triple product where a pure quaternion r is sandwiched between a quaternion and its conjugate

Let r= (0 e3), then

and describes at counter clockwise rotation of 2theta in the x-y plane about the z axis.  It's a pure quaternion, or just a regular vector.

We can also see that in general the quaternion sandwich preserves norm

which implies that .

To evaluate () for arbitrary q and r we will need to resolve the pure quaternion triple product

where we have used the vector triple cross-product formula from ().  Proceeding, and using (),

Writing () as a matrix times vector product provides a conversion to the standard rotation matrix

where

is the quaternion to rotation matrix.  There are other forms, for example using the identity in (lin alg section())

Now we are finally able to prove the triple product formula corresponds to rotation.  Define the rotation operator

() is linear

Next we express r in terms of components rperp and rparallel which match the geometry and then evaluate the effect of the rotation on each part in turn

Since rpar is proportional to the rotation axis v, rotation is the identitiy operator since it doesn't change length and we therefore know

For the perpendicular direction we evaluate () to obtain

where the unit axis is

and we have used v'rperp=0 and subsituted q=cos+sin*u..  For completeness we note that () provides yet another rodrigues formula from (section ())

rperperp has the same magnitude as rperp and is oriented perpendicular to both rperp and u as in figure ().  Eq () implies a rotation of rperp by an angle 2theta.

Figure () illustrates the full 3D picture which in equations is

Since r' and r are the sum of a common fixed component rparallel, and the perpendicular components rper and its image rperp' which are related by (), the effect of qrq* is to rotate r by 2theta about the axis q.

To rotate r about center c and direction u use

r' = q(u)(r-c)q(u)* + c

#### Derivatives

Using quaternions, R is parameterized interms of q as in ().  As in () the Jacobian of the to frame transformation with respect to q is (p is a regular vector)

Note that each entry defines a vector in terms of a matrix vector multiply and thus the (rk-r) cannot be factored out unless it is flatened.

The kth component (many forms), say starting from (), is using the chain rule (linalg section)

Further anyalytic simplification is possible by using tensor products.

To obtain the time derivative we write the updated quaternion as the product of its intial state times a small perturbation dq

This can be done because qtilde represents a rotation during the interval dt.  Same as rotation matrices (R(a+b) = R(a)R(b).  The perturbation can be linearized as in () as

Writing the definition of the time derivative and substituting () yields

where

In vector form

with .  The matrix

represents the mapping of  instantaneous angular velocity to quaternion rate of change.

#### Integration

To solve () we do so locally by Taylor expansion about a specific time tn.  The required derivatives are computed recursively from ().  Letting tn = nT then q(tn+T) = qn+1 and omegan = omega(nT),  the series solution during the interval n to n+1 is

The successive derivatives are

This method is identical to exponentiation.  During zeroth order integration, omega is held constant over the interval, hence omegadot=0, and

Defining the incremental rotation quaternion as

we can write the propagation as the matrix multiplication

We are free to choose any omega during the interval.  If the value of omega is received at the end of the period omegan+1 is backward integration.

#### Perturbations and Exponentials

Error dynamics of quaternions are important in filtering applications that separate a signal into large and small components.  Consider a small quaternion dq is applied to qhatresulting in the updated quaternin q= q0dq.  Note that this corresponds to a body frame application with multiplication from the right.  For global, the multiply is from the left which will result in a sign change in the final equations. We will also need the corresponding angular error dw = w - what.

where w and what corresponds to q and qhat, respectively.  The multiplying by the inverse of qhat we have

where we have used the definitions of quaternion derivatives (). Note omega is pure.  Carrying out the multiplication and restricting ourselves to the vector part in results in

Substituting in , leads to

Now using the small angle form of the quaternion results in

For a generic linear model we omit the quadratic errors to yield

Integegration proceeds by exponentiation to obtain the state transition matrix F = e^omega.  We use the same tricks as before: powers of skew-symmetric matrics and trigonmetric series to obtain

Therefore the exact solution, at least up to second order and with constant angular rate over the interval t to t+T, permits the discrete time update formulation with tn = nT

Using (), () can be used to approximate the angle transport as

With all the above we are now in a position to formulate both the state transition and measurement linearizations for inertial navigation.

#### Summary of Results and Useful Formulas

derivative of Omega(x)

di = Omega(ei)

#### Conversions

#### Lie Algebra and SO(3) Group

The notation and rules are convoluted, scalar plus vector.  Context will help.  In applications only need a 4-vector q or qk.

Requires 32 multiplications to rotate a vector as opposed to 9 for R.  It is slightly faster to convert q to R and then rotate.

#### Comparison

1) Avoids gimbal lock.  In the Euler formulation, rotations are sequential about fixed axes X,Y,Z.  If the middle rotation becomes aligned with one of the other axes, say X, then Y and X rotations are indistinguishable due to the loss of one degree of freedom.  Quaternions rotate around a single axis by one angle only so no gimbal lock is possible.  Also as a result of the single axis the is no dependency on ordering.

2) quaternions are continuous, therefore they can be added and interpolated without worrying about discontinuities at the boundaries of the range 0 to 2pi, or +/-180.  However, when extracting a quaternion from a corresponding rotation matrix, there are four choice and numerical issues can arise.

3) maintaining unit length on a quaternion is easier than maintaning orthogonality on a rotation matrix.

4) doesn't involve trancentendal functions, matrix is quadratic in elements of q

As we know, for any unitary quaternion

### Transformations

We will adhere to the convention:

r = Rp+r0 is from frame, where p is object moving frame and r is reference/global frame.  "From" body to world frame.

p = R'(r-r0) is to body  hence to frame.  This makes sense as an anchor because the body frame is the starting point for measurement.

For complicated situations with many coordinate frames to keep track off the notation  is used where alpha is the "to-frame" and beta is the "from frame.".  For example with p in body and r in world

Multiple transformations chain with the upper and lower indices "cancelling."  When applying a rotation, order matters depending on which frame the vector is in.  Thus for an application of a matrix C in world frame the matrix multiplies from the left in world space.

For a transformation in body frame it must come from the right because that is where body spaceis

R is the rotation matrix which expresses the attitude change from local to world coordinate frame.  In the figure, R is clockwise rotations.  The world point r = Rp  + t.  In the middle figure, it is only a change of basis.

Differentiating ()

illustrates the transport theorem for relative velocity expressed in world coordinates.  Equivalently, in body coordinates the velocity of rW in B is

## Bayes Probabilistic State Space Models

The joint distribution (p(x^n,y^n) contains all relevant information therefore application of Bayes' rule to flip conditiong assignments becomes a natural choice for a solution to estimate x given y.  Once we have the desired conditional pdf, we can select its mean as a point estimate.

The batch solution computes

p(x^n,y^n) = p(x^n |y^n)p(y^n) = p(y^n |x^n)p(x^n)

which yields the joint posterior distribution

p(x|y) = p(y|x)p(x)/p(y)

in terms of the measurement likelihood function p(y|x) = l(y) and a constant term p(y)

which does not depend on x.

Evaluation of () becomes possible when we forgo batch estimation and restrict our task to a recursive process of estimating only the current state x(n) based on all available measurements y^n.  We further assume a Markov sequence model for the evolution of state xn from state xn-1 so that p(xn|x^n-1) = p(xn|n-1) and conditional independence of measurements such that

p(yn|x^n,y^n-1) = p(yn|xn)

Under this set of restrictrions we require:

1) An apriori state x0~p(x0) because we need a starting point for moving from n to n-1 when n =1.

2) A dynamic model xn~p(xn|xn-1) which provides for state transition between time steps

3)A measurement model yn ~ p(yn|xn) which provides a noisy information correlated with the desired state

4) Predictive posterior distribution p(yn|yn-1) prediction of current measurement given previous

The recursive solution proceeds from Bayes's rule identity in section (Multiple RV) by block partitioning the data at time n .  In the context of filtering n is viewed as the "future" or update time, n-1 is the present time, and n<n-1 is the past.  The filtering posterior distribution updates x given all measurements is

where the denominator

is independent of x and hence can be neglected and we have used the conditional independence of measurements assumption.  The so called propagation term is derived by computing the marginal distribution p(xn-1|y^n-1) from the joint conditional distribution p(xn,xn-1|y^n-1) and invoking Markov property of the sequence x(n)

which is seen to be the product of the state transition model and the previous time posterior and is known as the Chapman-Kolmogorov equation.

Letting

be the measurement model, the filtered posterior, and the transition model, respectively.

Combining () reinforces the recursive nature we obtain the desired posterior conditional distribution

Thus there are two steps for computing (), the prediction or update step computes a weighted average of the previous posterior distribution based on the transition model, and two, the measurement model (also known as the likelihood function) is used to modulate the prediction.

This is the canonical starting point.  General solution, though not always implementable due to complexity or nonlinear functions.  Starting point of all approximations.  Where possible point estimate  can be obtained from the distribution according to MAP or MMSE.

### Conditional Dependencies

Individual measurements often exhibit sparse dependencies in the state model.  With fixed observed features or landmarks, bearing measurement b(i,j) between frames i and j

### Tracking Back-Ends Overview

KF - linear, gaussian optimal

EKF - as good as linearization, sensitive to errors beyond 20deg.  Approximates pdfs with Gaussian mean (NL function evaluated at mean) and approximate linearized covariance evaluated at mean. After linearization and Gaussianization, conditional mean of joint pdfy [x z] is formed with approximate moments.  Particle filter with one particle.

iterated EKF-relinearize about the filtered estimate a few times.

performance depends on quality of linearization.  If linearization is performed far away from from true estimate breaks down.  EKF approximates pdf of nonlinear transformation by gaussian with non-linear mean and linearized covariance.  These can be biased.  No guarantee distribution is symmetric or single mode, e.g. skew could be non zero.

K^2 cost per step, K memory, cubic complexity from 3+2K inversion

UKF

approximates mean and covariance with preselected sample sigma points.

nonparametric.  more like least squares with sample matrix.  estimate mean and covariance with sample points (sigma points).  chose 2N+1 points for both mean and covariance.  points clustered around mean in N dimensions.  Sigma points work well if they are close to location where nonlinear function is evaluated.

Propagate weighted sigma points through nonlinear function, compute mean and variance only.  Equivalent to KF for linear.

somewhat slower than EKF, differences somewhat small compared to EKF

Extended Information filter

Replace sigma by inverse (information matrix), mu = inv(Omega)*xsi,

Sparse extended Information filter

can get constant time

eliminate links in covariance, active (linked to pose) and passive (no link to pose).  Measurement adds new element to information matrix.  As robot moves all landmarks become correlated, sparsification step pairs down these off diagonal elements.  only pair active landmark, eliminate link between pose

1. state prediction, 2. measurement, 3. state correction, 4. sparsification.

Sparsification: replace full distribution with pdf conditioned on single variable only.  Divide LMs into 3 classes m+ (active), m- (passive), m0 (active become passive)

For state correction do it sparsely only for robot state and active LM, ML estimate mu'Omega*mu +

Particle filter - approximates distribution with randomly selected sample points and weights.  Particles drawn from q to approximate p, with w=p/q.  Weights determined recursively.

non gaussian, non linear

Monte Carlo localization, each particle is a pose hypothesis, apply motion model, compute weights which approximates p(z|x,m), work well in low dimensions because of need more random samples for high dimension

FastSLAM 1.0 - lth  particle filter respresents pose, and corresponding landmark map.  kindof MHT  Rao-Blackwellization p(x,m,z) = p(m|x,z)p(x|z), where p(m|x) is easy to calculate, x is lowdimensional

in graph, with known pose landmarks are independent

Rao-Blackwellization for SLAM

each particle is robot pose, 2x2 Kalman filters for each landmark

weights are from expected observation distribution Gaussian

target p(xt|zt), proposal p(xt-1,zt-1

proposal distribution based on controls

FastSLAM 2.0 - proposal dist includes measurements as well.

### Statistical Sensor Fusion

## Filtering with Gauss-Markov Linear Models

Eq () cannot in general be computed, however, under linear Gauss-Markov models the optimal state estimate is the Kalman filter which computes a closed form recursive conditional mean.    In this case the prediction integral is solved by properties of Gaussian random variables.  In particular, from (section ()) we know that if y is an affine function of x and x is N(mu,Sigma) then p(y), p(x,y) and p(y|x) p(x|y) are all normal.  When we move from estimation of a constant parameter to a random walk state model we enter the filtering domain where we iteratively estimate the desired state based on the following measurement and propagation model

where .  Under such a model we can immediately determine the required input distributions.  For the prediction step in the Chapman-Kolmogorov equation we know (GRV section ()) if y is an affine function of x, p(y|x), p(x) and p(x,y) are Gaussian with closed form moments..  In particular,

where we have defined

as the filtered estimate and error covariance.  To evaluate the predicted estimate in the CK equation, we know the input densities p(y|x) and p(x(n-1)|y^n-1) are normal, hence so is p(x(n),x(n-1)|y^n-1).  Actually, p(x(n-1)|y^n-1) is strictly normal only if it is initialized to p(x(0)) = N(mu0,P0).   Using the state model we find the prediction moments are

For the joint distribution, we also need the conditional cross covariance

The joint distribution is then

and the marginal is

In the filtering stage, the predicted moments are used to update the filtered moments upon receipt of the new measurement y(n).  Again, we know p(y(n)|x(n) and p(x(n)|y^n-1) are Gaussian, hence so is the joint distribution p(x(n),y(n)|y^n-1).  Plugging in the above values for the mean and covariance of x(n) and y(n) yields

Recall, the prediction moments are used because of the conditioning on y^n-1.  The updated conditional posterior is (Theorem Bayes section () GRV),  where the conditional mean and error covariance are

S(n) is the measurement covariance.  K is the Kalman gain which is inverse to the measurement uncertaintywhich makes the correction term large for good measurements and small o/w, e.g. biases towards the prediction..adaptive bandwidth for measurement acceptence.

## Graph Least Squares

Basic nonlinear least squares described in section MA section ().  Typically used in vSLAM to minimize reprojected error of key points.  In mono bearing only line with depth unknown, in stereo range bearing.  minimize ei^2 = (zi - f(xi))^TSigmainv() as a function of x.No easy general solution, due to presence of local mins.

Need good initial guess and smooth differentiable function.  ei(x+dx) = ei(x) + Jidx

Ei = e'Ae, min sum(Ei) = quadratic form in dx,  take derivative, dx = Ainv*b., xn = xn-1 +dxn.  MSE is same as ML for gaussian

constraints connect the poses of the robot while it's moving

observing previously seen areas generates constraints between non-consecutive poses

node = pose, edge = spatial constraint based on correspondences between features.

xi is pose, xi to xj related by constraint which is a spatial transformation, e.g by odometry or matched camera views

Back End Graph Optimization

## Continuous Time Linear State Equations

Solving specific differential equations in closed form involves clever tricks because no general analytic procedure exists.  The homogeneous (zero input, uncontrolled, etc) linear time varying differential equation

due to linearity is guessed to have a unique linear solution at time t based on initial condition x(0)

which by construction implies P satisfies

Which depends only on F and requires P(0)=I so that x(0)=P(0)*x(0)

Evolving from intial condition x(t=0).  Equivalently we may write the solution in terms of another time tau by solving for the initial condition

The nonsingular matrix P is known as the fundamental matrix.  () may be succinctly stated in terms of the state transition matrix which we define as

Where the transition matrix, among other things, is subject to

Differentiating

Aside from some convenient properties on F we have made little progrees, and indeed unleass A is diagonal, triangular, or constant during the transition interval.  In view of an eye towards discretization in the interval [tn-1,tn] let us make such an assumption on F,  .  Then  becomes

Which is easily integrated by separation of variables

Where we have used the initial conditions in  and  .

Then the solution to  is

The matrix exponential in  is defined for any T as the infinite series

For sufficiently small value of the interval

Hence,  may be approximated as

Which amounts to the zeroth order indtegration of .

Another equivalent solution is to locally expand on the interval in a Taylor series.

If there is a constant term b

If A is invertible,

An added benefit of this closed form solution is the easy Taylor expansion

### Example: Coordinated Turn

The following presents the coordinated turn model in Cartesian velocity formulation which results in a linear state equation if the angular velocity is known or controlled.

From the general planar kinematic equations in normal-tangential coordinates described by the unit vectors et,en we have

In a coordinated turn is a special case of circular motion in which the angular velocity (bearing rate) is constant, and the speed (velocity magnitude) is constant,  , while only the bearing changes.

Denoting  the state vector for the system is we have the state space representation of  and  is

Where the continuous time transition matrix is

The constant turn radius is given by the equations of circular motion

Where K is the familiar skew symmetric matrix describing a lossless mechanical oscillator.  The characteristic function is

, therefore the eigenvalues are .  The eigenvectors are found to be

The discretized transition matrix is therefore

For the states  with nearly constant turn rate the state equation is nonlinear in omega

With state model

Linearizing to first order through the Jacobian

### Example: Nonlinear State Equations

Here we present the method of direct integration which sometimes works.

In polar coordinates the coordinated turn state vector is

with corresponding dynamic state vector  related nonlinearly by .  In this case direct integration provides an exact discretization in the form

where g1 and g2 integrate the position, g3,g5=0, and g4 is linear.

The integration depends on the linear relationship between bearing and bearing rate  and proceeds via the following steps using only change of variables and trigonometric identities.

Eq () can be written in terms of Tsinc(wT) functions or further simplified (matlab) to

where as usual and which reduces to linear motion for .  The above steps can be repeated for x to yield

The matrix representation for the combined non-linear state equation is

which can be linearized for EKF state propagation.

## Range-Bearing 2D EKF Localization

A 2D model with range bearing measurements a priori extracted from detected features is quite simple and illustrates the basics of localization without requiring 3D pose estimation, feature extraction, complicated state dynamics or explicit frame conversions.   In localization only, e.g. not mapping as well, we assume known landmark locations in the world frame.

The discretized polar coordinated turn model described in () is used for state dynamics.  This choice results in both non-linear state and measurement equations which are linearized about the predicted estimate.  It is also possible to use a Cartesian turn model which results in linear state propagation for known turn rate, however performance can be shown to be superior in the former case.  The robot course is governed by control inputs u = (v,w) which serve as known inputs to the state estimator where deviation from truth is accounted for in the measurement noise parameters.  Since the controls are assumed known they are not required to be incorporated into the state vector, however it's possible to achieve better performance by their inclusion.??. State variables are computed in the world frame wrt a suitable origin (such as the robot's initial position) and measurements in the sensor frame are converted to the world frame inside the filter.

On board instruments provide local frame range bearing measurements (rhok,psik) to K observed landmarks at world frame positions  relative to the robot/target heading. Typical sensors are laser range finders, LIDAR arrays, or stereo cameras.  For now we assume perfect data association which means the kth measurement corresponds exactly to the kth landmark.

Figure () illustrates the local frame measurents have bearing measured with respect to the current heading direction and range relative to the current robot position.

The noise free system state model for

Noise is added and then is discretized and linearized to

Differentiating ()-() results in the linearized zero bearing rate state matrices, respectively

For process noise we select constant normal and tangential acceleration a, alpha during the sampling interval which are manifested as zero mean normal random variables.  The function of the process noise is to model the uncertainty in the state estimate whose dynamics do not include acceleration, and thus the filter can only be made aware of such motion through external imposed uncertainty. Substituting

into () and attempting a direct integration as in section () results in a very complicated nonlinear expression.  Another possibility is to assume constant heading between sampling instants, in which case the normal acceleration adds the required uncertainty term as a function of the unknown acceleration a.  Then using ()-() in the direct integration additional terms

and linear terms

Therefore we can write the complete state equation including an additive dynamical uncertainty as

where

and

with total process noise covariance

The other possibility is to expand the nonlinear portion of () in a first order Taylor series about the previous time step and use the differential terms as the residual acceleration uncertainty.  This procedure propagates the uncertainty from control space to state to obtain the state space.

We have

Thus the process covariance can again be written in terms of a G matrix containing partial derivatives with respect to the uncertain control variables.  Range and bearing are measured wrt vehicle reference frame therefore the world coordinate measurements are

Since atan2 has a discontinuity  +/-180 deg, it is important to check the innovations operation and unwrap the angle.  As the noise increases the bearing can oscillate close to the discontinuity.  Or if the true bearing is constant at say, 359 deg, noise will cause frequent pi changes.  When adding angles it is important to check to make sure the sum is in the correct range, say 0-359 or -179 to 180.

Linearization matrix is for the kth measurement is

where

This can also be written as

The measurement covariance accounts for noisy sensor outputs and is assumed to be zero mean Gaussian, e.g.

Measurements from each sensor are processed sequentially to obtain successive state corrections.  Due to independence, for a linear filter the order does not matter.  However, for non-linear filters small improvements can be made in linearization if more accurate sensors are processed first.

Initialization

Simulation results

#### Simulated Results

K=50, heading error 2 deg, range error 2 m, T=1s

## Range-Bearing 2D EKF Localization and Mapping

A common application in robotics is navigation in an unknown environment.  Now, the landmark locations are unknown, e.g. the map, as well as the target state, e.g. the pose.  To estimate the map, we now include the K landmark positions () in the state vector which has dimension 2K + 5 (or 3).  In this development, we will assume all K landmarks are observed at each n.  However, landmarks can also be added when new ones occur.  Some heuristics and theory describe procedures for this.  Again we assume perfect data association.

The state vector becomes

and the error covariance is

The landmarks are assumed fixed, so no motion model is required.  Same as localization.

The landmark locations are initialized by inverting the measurement function and substituting the current robot pose from the predicted estimate as values for x,y and theta.  Thus we have

for the predicted initial landmark locations which used in the measurement functions, h and H.  The measurement function is extended to include the landmarks and subsequent jacobians.  The measurement linearized matrix must now include in addition to the derivatives of pose the derivatives wrt the landmark positions which are identical but opposite sign to () and thus will have blocks

Setting the origin to be the location of the robot at t=0, we can initialize the heading to zero.  The coordinate system will then be with respect to the initial heading.  Alternatively, an external sensor can be used to set the heading, say wrt to North.

Data structures can either be worked through large sparse matrices or expansion upon measurement receipt.  Prediction is order K complexity, and correction is order K^2 for inverting a K x K symmetric matrix.  The EKF can be used to estimate the state.

### Simulated Results

## 3-D Visual SLAM

Stereo cameras function as range-bearing sensors thus with a number of landmarks a least squares simultaneous solution for robot and landmark positions is possible.  The known camera baseline provides scale to the problem which allows for distance estimation.  Monostatic cameras combined with odometry replicate a stereo configuration with the odometry providing the predicted baseline.  Motion must be such that it creates a parallax, ideally perpendicular to bearing line.  If the robot position is known, the landmark position can be estimate from two bearing angles, the displacement between bearing views via law of sines.

In Mono-SLAM since only attitude and position are visible during correction there are various gauge freedoms which apply.  For example rotation or translation of both frames results in an indistinguisable configuration.  Also universal scaling in the projection function cancels out, hence the camera measurements remain unchanged for a a scale change.  The addition of an accelerometer removes scale gauge freedoms because it is calibrated in metric units, e.g. m/s^2.

In stereo, get range and bearing measurements to features throughout the image

In vision systems, z is the typically the direction of travel optical axis, whereas in tracking z is vertical. To accommodate this a rotation or axis swap (y to z or x to z) is made by a rotation matrix

Therefore, when constructing the camera matrix P, Rbc premultiplies R.

A robot navigates in an inertial world frame W observing visual landmarks pk in the camera frame C (there can be other frames which necessitate additional transformations).  The location of a point rk in W corresponding to pk in C is given by the from frame or forward transformation function

where R rotates C to W and r defines the robot position in W.  Eq () defines a triangle composed of the long side rk (in W), the vector Rpk (in W) and r (in W).  R is parameterized by the robot attitude in terms of Euler angles, quaternions, or axis-angle representation.  The to frame or backward transformation function converts world/navigation/reference frame to local measurements via

In () the local position vector is first translated to the World origin then oppositely rotated to align with W.

You can add sub and superscripts to help keep track of the frames, however, for two frames its easier to draw the picture and use the from frame transformation as the base transformation.  Then for example the the translation of the origin in C-frame is r (in C) = -Rt*r

The state vector

is minimally comprised of the attitude psi or q, the robot position r, and the landmark positions rk all in the world frame.  There are a few basic parts to VSLAM: state model linearization, measurement linearization, and optimization method.  First we discuss the measurement models for mono and stereo cameras. Then state models, and finally two front end parameter estimation schemes will be discussed , EKF and least squares iterative search.

The light ray emanating from a 3D feature point pk is projected onto the image plane of the camera and converted to a 2D pixel value yk = (uk,vk) by the photodetector.  See section (computer vision) for details.  If there are K landmarks, the 2K dimensional measurement vector contains pixel coordinates for each feature location

The relationship between the measurements and the state vector evolves in two steps: 1) transformation from rk to pk, and 2) pinhole projection via

.  The final compound function h(x) is hc(hw(x)), where h relates states x to observables y, hw is the inner function representing frame transformation, and hc is the outer function describing pinhole projection of the camera frame landmark 3d position.  All of these functions are nonlinear wrt some portion of x.  The linearization for an EKF of non-linear least squares parameter estimate thus requires the overall first order partial deriviative matrix H which is computed via the chain rule for Jacobians (optimization section)

HW maps the 6 + 3K state vector to 3K x 1 camera frame landmark positions.  HC maps those positions into 2Kx1 measurement vector representing image plane feature locations.  The Jacobian propagates the nested derivative information as

These matrices, which are sparse, determine and propagate the relationships between measurements and states.  First, let's compute the Jacobian for the inner function hc of the kth landmark, remembering the function structure when laying out the matrices.

where f is the normalized focal length (in pixels, f/pixel dimension) and (u0,v0) is the image center in pixels.  The Jacobian matrix maps a 3D point pk = (xk,yk,zk) to a pixel location yk = (uk,vk) so it has dimension 2 x 3.  It is independent of all other state information r and psi and other landmarks.

For K landmarks HC will be block diagonal in HC(pk) and will have size 2K x 3K.

Next we turn our attention to the the rigid body transformation function for a single point rk which we write in terms of scalar functions as

where

and ci is the ith column of R.  This function depends on robot frame parameters r and psi, as well as the the landmark rk.

From the definition of the Jacobian, and with each element a derivative of a scalar function wrt a vector,

with

and Dk representing the attitude derivative.  Also note the transpose in the denominator of the partial derivatives to preserve ordering.  The total size is 3K x (6 + 3K).  Each scalar derivative wrt a vector is 3x1.  The only tricky part here is differentiating wrt the angle representation.  First we will use Euler angles, and then exhibit a compact representation for quaternions.  When R is expressed in terms Euler angles the derivative wrt to attitude is

For example

Finally,

For K = 3, HC*HW*x = (2K x 3K) x (3K x 6+3K) x (6+3K) = 2K x 1

() can be partitioned as

However, in EKF SLAM measurements are processed sequentially, so only the single landmark for is required. The measurement matrix is a function of the states, hence one must subsitute the most recent filtered estimate xhat in order to evaluate it numerically.

Using quaternions, R is parameterized interms of q as in ().  The only thing that changes is the computation of the Jacobian of the to frame transformation with respect to q.

The kth component (many forms), say starting from (), is using the chain rule (linalg section)

Note that each entry defines a vector in terms of a matrix vector multiply and thus the (rk-r) cannot be factored out.  By brute force we have

A compact form can be obtained by matching terms

The derivative can also be computed by brute force we have

D =
 
[2*q0*r0 - 2*q2*r2 + 2*q3*r1, 2*q1*r0 + 2*q2*r1 + 2*q3*r2, 2*q1*r1 - 2*q0*r2 - 2*q2*r0, 2*q0*r1 + 2*q1*r2 - 2*q3*r0]
[2*q0*r1 + 2*q1*r2 - 2*q3*r0, 2*q0*r2 - 2*q1*r1 + 2*q2*r0, 2*q1*r0 + 2*q2*r1 + 2*q3*r2, 2*q2*r2 - 2*q0*r0 - 2*q3*r1]
[2*q0*r2 - 2*q1*r1 + 2*q2*r0, 2*q3*r0 - 2*q1*r2 - 2*q0*r1, 2*q0*r0 - 2*q2*r2 + 2*q3*r1, 2*q1*r0 + 2*q2*r1 + 2*q3*r2]

s2 = [q0;-q3;q2];

s3 = [q3;q0;-q1];

s4 = [-q2;q1;q0];

s1 = -[q1;q2;q3];

S = [s1.';s2.';s3.';s4.']

A = S*r

a1 = s1.'*r;

a2 = s2.'*r;

a3 = s3.'*r;

a4 = s4.'*r;

Then () is

[a2 -a1 a4 -a3;a3 -a4 -a1 a2;a4 a3 -a2 -a1]

which is local mapping between differential quaternion and angles.....Zeta matrix.

## Data Association

Match measurement to landmark through innovations nearest Mahalanobis distance from kth landmark to lth innovations, e.g. select kth LM and the loop of l=1:K measurements.  For conditionally independent measurements, the pdf of the measurement is normal with mean zhat and covariance S.  Hence the ML estimate of the corresponding measurement is

which is a function only of the lth measurement.  However, two closley spaced measurements with large errors it's quite possible for errors in association.  Heuristics such can be used.

Which measurement kth belongs to the mth landmark? easiest: pick m, loop over k and find ML estimate with innovations pdf.  With perfect data association the correspondence matrix C is diagonal with cij =1 for i=j, and 0 otherwise

Pose influences data association.

A joint distance set can be used as well, where the individual innovations for the set of measurements is stacked and gated together.  Joint compatibility branch and bound using search tree.

## Loop Closing

Loop closure is built in for perfect data association.  Once the landmark is observed the filter embeds the correlation between its position and the vehicle state into P.  When mj is reobserved, the residual updates the states and the map jointly.  In pose-graph slam, once a landmark is reobserved (e.g. it should have the same x,y,z since it's stationary), the error between the previous and current pose is redistributed among the other poses via optimization.

## Inertial Navigation

Inertial navigation systems combine sensors which measure acceleration and rotation rate with equations that solve Newton's dyamical equations of motion to compute a body's position and velocity.  The advantage of such systems are their self contained nature, requiring no external signals or input.  Mechanical systems are mounted on a stable platform which is isolated or gimbaled from the vehicle's rotational motion.  For applications which do not require extreme accuracy most modern systems dispense with the heavy and expensive mechanical appartus and employ microelectronic or optical sensors which are rigidly attached, or "strapped down" to the vehicle.

Integrated inertial measurement units (IMU) combine three gyroscopes and accelerometers which provide rotation rates and non-gravitational acceleration, respectively, along three mutually perpendicular axes.  IMUs may also include barometers to measure height and magnetometers to measure compass heading.  Gradiometers can be used to measure the gravity direction.  MEMS are sensor systems composed of electrical and mechanical parts usually etched out of silicon at the micrometer scale. Mems accelerometer spring etched in Silicon, motion of proof mass changes capacitance.  MEMS gyroscopes have a mass that reciprocates at a constant frequency. During rotation of the gyroscope, the mass will induce a force perpendicular to the reciprocation direction due to the Coriolis effect. This force is countered by the etched springs and sensed by capacitive sensing arms like the accelerometer.  Typical errors in industrial MEMs are few degrees per hour and a few micro-gs in the gyro and accel which results in accurate dead-reckoning for a few minutes.  Strategic platforms feature errors up to 10-4 deg/hour which is suitable for a few hours of accurate course estimation before a corrective input is required.

Given knowledge of intial speed, heading, and position it is theoretically possible to determine subsequent values via dead reckoning.  The gyroscopic measurements are integrated to determine attitude and heading which are then used to resolve accelerometer measurements into the desired reference or navigation frame.  Integrating the oriented accelerations twice provides velocity and position with respect to this frame.

Another advatange of VIO-SLAM is the addition of an accelerometer resolves the scale gauge similarity.  For situations where a=0, scale is unobservable.  With known focal length, the camera measures only bearing.  It has no ruler, because the projection function is invariant to scale.  However, a calibrated accelerometer allows the perception of metric distance since its units are m/s^2.  The scale of the world map is constantly informed and corrected through the predicted measurement which uses the metric propagation model.  If a synthetic scale change is added to the map, the innovations process becomes non-zero, and the gain adapts the correction to enforce the metric scale maintained by the accel.

In principle then there are two main steps to inertial navigation: 1) Use the gyro data to orient the accel values into the reference frame, 2) proceed with dead-reckoning in the reference frame.  These two fundamental operations are encasulated in the navigation equation

The position is required in frame-alpha, the accel f and rate w outputs are in frame-beta, the rotation matrix R resolves from beta to alpha, g is the known gravity vector in alpha frame and can depend on r.

### Attitude Update Computation

For systems with small  and to first order  can be approximated by

which for a single angle rotation, say  , is equivalent to the linear update given by .   Note in other texts, the definition of the skew-symmetric matrix can differ by a factor -1 which leads to alternate representations.

### Acceleration Update Computation

Accelerometers measure total acceleration including gravity.  Holding an accel stationary will result in a reading of +1g because an upward force is canceling the downward gforce.  An accelerometer can be modeled as a small proof mass attached to a spring.  For a single vertical axis, the force on the mass is given by Newton's second law

where k is the known spring constant, u is the measured displacement along the sensitive axis, m is the known mass, g is the known or stored gravitational acceleration, and a is the acceleration of the mass.  When the vehicle is stationary, a=0, and the measured displacement u0= -mg/k is that required to support the weight under the force of gravity.  When the spring unit is in freefall, a=g, and u=0, thus the ouput will be null.  When the vehicle is accelerating we have

where f is the non-gravitational, or so called specific force, which is what the accelerometer actually measures.  Thus with the addition of local gravity information, from () we can determine the acceleration

which is simply the difference between the measured response and gravity.  Implementation of such spring like systems can be done with motion induced capactance changes or via laser interferometry.

Therefore with error free uniformly accelerating 1-D motion perpendicular to gravity and knowledge of initial conditions, velocity can be found by integrating the sensor output

and the updated position can be found by integrating the velocity

In reality, various forms of error such as drift and noise render this approach impractical.  Furthermore, the acceleration measured in the sensor frame must be transformed into the navigation frame using the measured angular rates which are subject to drift and noise as well.  A common approach is to integrate both gyro and acc data, as well as the corresponding drifts into a Kalman filter.  Even better is to add sporadic independent position fixes, such as by GPS, into the filter to correct for sensor errors.

The integration formulas are valid for small update intervals.  As the update interval increases relative to dynamical changes, more higher order integrate techniques are required: trapezoidal, 2nd order, etc.

In two dimensions two accelerometers (axb,ayb) and one z-axis rate gyro are required.  After estimating the attitude by integrating the rotation rate, the accel specific force must resolved the sensor/body frame acceleration into the reference axes.

For position determination in an inertial reference frame the update equations are 3D analogous to () with the addition of a coordinate transformation and accounting for gravity.  Once the specific force is resolved into the inertial frame as

In three dimensions, the attitude transformation R(n) is computed by integrating the gyro output and is given by ().  The intertial frame acceleration is

and the position is

### Initialization and Calibration

For acceleration without a linear component the initial pose may be estimated from the known gravity vector .  This is known as alignment.

The heading can be estimated from a magnetic compass.  Other possibilities include initialization based on external information such as GPS, location proximity, or radio positioning.

## Vision Aided INS with ESKF

INS and GPS systems exhibit complementary advantages and disadvantages.  INS features high update rates, stealthy independent operation, and low short term noise.  GPS is characterized by low update rates, signal dependence, and high short term noise.  GPS is continuously accurate to around 1 meter while INS requires accurate initialization and performance degrades overtime.  The degree of integration can vary between simply using the external data to reinitialize the INS to having deep control of the base signals for example GPS timing, psuedo-range, etc.  Since accurate INS are expensive, the combination of a cheaper INS with additional sensors is an attractive architecture.

The INS state propagation equations are fused or integegrated with measurements derived from additional sensors such as GPS, vision, LIDAR, radar, odometry, compass, etc.  These additional measurements provide absolute position information which can be used to compute INS biases.  Two types of filters will be discussed: the standard EKF filter, in this context known as the total state Kalman filter (TSKF) and the Error State Kalman Filter (ESKF).

For the TSKF you have INS based state prediction, followed by psuedo position measurements which correct the INS values as they drift over time.  In ESKF, you have total state INS based prediction, followed by a separate error only filter which corrects the prediction.

In general aiding measurements integrated into the ESKF are y = h(x). These measurements may consist of range, range-rate, and direction or direct position and velocity either from a radar or GPS.  The remainder of the section discusses the use of a camera which detects salient features in the field of view.

By detecting and matching features between images captured by a camera at two or more different locations the egocentric motion of a body can be determined.  This ego motion consists of estimating the pose (rotation matrix) and distance (translation) between the different views.   here are two types.  Stereo and Mono. Similar to SLAM with the addition of measured angular rate and acceleration.

In monostatic camera systems, a single camera provides bearing measurements where each pixel represents a cone of light passing through the lens optical center.  Since pixels are small, the angular resolution is generally high and for large format sensors combined with wide angle lenses the field of view is large as well.  Depth information is directly unobservalble due to the projection of features onto the sensor which is located at the focal length of the lens.  However, both pose and feature location can be recovered from images taken at multiple viewpoints and locations. Without direct range measurements the metric scale is also unobservable.

A camera is bearing sensor where the line of bearing to the feature is through the lens center and terminating at the photo detector located at (x,y,f) where f is the focal length.  With the conventions defined in section 1.1 such that z is aligned with the optical axis and the focal plane resides at z=f in the x-y plane, the direction unit vector is

In bearings only target motion analysis, e.g. when only the azimuth angle is considered, () reduces to

A monostatic camera functions as a bearings only sensor.  Parallax triangulation becomes possible when vehicle motion generates a baseline separating mutliple views.  The pixel location x is corrupted by zero mean noise sigma^2 due to optical and signal processing limitations in detecting the precise feature location..  Expanding (above) in a linear error model and assuming the feature is located near the optical axis, yields

The bearing variance is then

The error-state Kalman filter (ESKF) uses three conceptual versions of the same physical state: the unknown true state, the stored nominal state, and a small local error state. The true state is related to the nominal state by a local perturbation. For Euclidean quantities such as position, velocity, biases, and landmarks, this perturbation is additive. For attitude, it is multiplicative:

\[ x_{\text{true}}=\hat x\boxplus\delta x, \qquad R_{\text{true}}=\hat R\operatorname{Exp}([\delta\theta]_\times). \]

The nominal state is propagated through the full nonlinear kinematic model using the IMU measurements, corrected by the current nominal bias estimates. The local error state represents the unknown difference between the nominal and true states. It is not necessarily high frequency: for example, a small accelerometer-bias error can cause slowly growing position and velocity errors.

The filter propagates the covariance of this local error through a linearized error model:

\[ \delta x_{k+1}\approx F_k\delta x_k+L_kn_k. \]

Here, process noise \(n_k\) drives the error state but is not itself the error state. Camera, ranging, or other measurements produce an innovation between the actual measurement and the measurement predicted from the nominal state. The Kalman gain uses that innovation to estimate a local correction:

\[ \delta\hat x=K(y-h(\hat x)). \]

Attitude estimation is best performed in the quaternion domain because they are less susceptible to gimble lock.  Rotation quaternions are necessiarly unit norm, hence applying the standard additive correction violates the unit norm constraint leading to radial errors in the quaternion.  Furthermore, the error covariance of a rotation quaternion is fundamentally less than 4 because the unit norm constraint provides only 3 degrees of freedom.  Therefore, the correction pipeline operates with attitude angle errors instead of a 4-D quaternion comprising 15 dimensions instead of 16..  Once the attitude correction is estimated it is injected into the state vector to update the quaternion portion.  The angle covariance is updated by a reset transformation to account for the new attitude.

One can see the singular nature of an additive quaternion error covariance by showing that an allowable quaternion perterbation must be perpendicular to the estimated q.

The error covariance is

which proves that qhat is a null vector eigenvector of Pq, and hence Pq has only 3 nonzero remaining eigenvalues, e.g. it is singular.

Below we present the basic estimation procedure assuming constant angular velocity per time step with zeroth order integration.  Higher order or midpoint integration can also be used, especially for high update rates and target motion.  It becomes more complicated if you estimate camera extrinsics, time offset, rolling-shutter effects, lens distortion, inverse-depth landmarks, or dynamic landmarks. used, especially for high update rates and target motion.  Also, we assume perfect target association.  This means feature k always corresponds to landmark k.  In general feature matching and data association must be performed prior to assigning an image feature to a physical landmark.  One simple but non-optimal approach is to select the feature index which minimizes the normalized squared error for the current measurement value.  More complicated methods keep several filters (hypotheses) running in parallel for different data associations.

### Propagation and Prediction

The state vector contains 3D position and velocity, a 4D body frame attitude quaternion, and 2 3D body frame biases for the gyro and accel.

for a total of 16 states.  The corresponding 15 dimensional error states are

The noiseless continuous time state propagation model is

Note that in (0, omega is understood to represent a pure quaternion when required.  Within the interval  all dynamical quantities are assumed constant.  The solution to ()-() as before is via local Taylor series in the interval t to t+T, or matrix exponential.  () is an exact solution for constant angular velocity as described in 22.2..4.4.  This assumption permits a very simple state transition model:

The   are the local accelerometer and gyroscope sensor outputs and  are the corresponding biases.  We use z instead of u to because the inputs are more akin to measurements than controls.  The matrix R resolves acceleration into the global frame and g is the gravity vector which for now we assume known.

The estimated biases are explicitly subtracted from the IMU inputs such that state propagation and measurement correct are functions of the bias corrected inputs

The ESKF can seem confusing.  It helps to start from first principles beginning with the prediction step.  A standard EKF produces a linearization for the state function evaluated at the previous filtered state estimate xhat(n-1|n-1). The predicted estimate at time n is xbar = Ef(xhatn-1)|Y).  The purpose of the linearization is to enable computation of the predicted error covariance.  To first order the Taylor series expansion about the previous filtered estimate is

where the state Jacobian matrix is

Defining  and   as the predicted and filtered state error, respectively, () is equivalent to

The resulting predicted error covariance is

Recall, the main purpose for prediction is to compute the apriori error covariance.  A complication arises here because quaternions produce a covariance matrix with a redundant degree of freedom.  There are 4 states, one of which is constrained by the other three resulting in a non-singular covariance matrix.  (put in derivation).  The remedy for this is to maintain q for nominal state estimation due to its robustness to gimble lock, but compute the covariance using a body frame angle perturbation .Mu is composed by a direction and a small angle.  For example, mu = [eps 0 0], represents a small rotation around the body x-axis.It turns out, a small directional perturbation is the natural coordinate frame for 3D attitude, known as the local tangent space.

State prediction is done using the full 16D quaternion model.  All portions of the filter involving correction use 15 states, including covariance prediction, state correction, and covariance correction.

The Jacobian must be computed with respect to the covariance coordinate system, e.g. in mu space.  Using x = xhat + deltax we reformulate (taylor series) as

where now by the chain rule

F is the derivative of the predicted error wrt the previous filtered error.  For additive states

but if the error is subtracted, dx/dxtilde will invert.  Eq () provides a prescription to numerically compute the derivatives by letting xtilden-1 = eps.  For the ith component of f, the numerical equivalent of () is

where the circle addition and subtraction apply multiplicative equivalents for the body frame attitude error.

For all error states except the attitude, the error propagation Jacobians are equivalent to the nominal values.  This is because the non-angle error coordinate is a translation of the predicted state, so its Jacobians are identity.  Techically, for non-additive errors, e.g. attitude, the plus and minus signs indicated in the LHS of () should be replaced with a special symbol to accommade multiplicative errors.  Many notations, such as box plus and box minus, attempt to accommodate this result by defining addition as normal addition for non-angle terms, and an SO(3) mapping for attitude error.  Note that we use mu, instead of theta to indicate the angular error term is an axis times a small angle.

To compute Fmu we require a recursion for delta mu.  All angular quantities (attitude error and gyro bias) will be expressed in body frame.  For this convention the small correction rotation

acts on the estimated rotation via

(12)

and thus

Differentiating () yields

which is an exact expression.  Applying the first order approximations for Rtilde and neglecting error product terms, and substituting the definition  we arrive at

Finally equating arguments we arrive at the continous time equation for the attitute error

What remains is to convert the constant coefficient differential equation () to discrete time (note that omega is assumed constant over the interval T.  Following the standard exponentiation procedure in (optimization theory section 4.5) yields

Since Omega is not invertible the right term must be integrated directly.  After changing variables and substituting Rodrigues's formula for the exponential, the integral can integrated term by term to arrive at

where

This expression is composed of two separate errors which independently reflect a misalignment in attitude.  The first term is purely a coordinate system error.  Even if the gyro and nominal rates are equal, if mutilde ~-0, then the relative frame is fixed, but the wrong label is applied.  The bias turn is a physical rotational error approximately proportional to the IMU time interval T.

With the above background and equiped with the machinary from 22.2, we are now in a position to compute the elements of F.

The Jacobian blocks for Fmu are read off from (error equation)

To find derivatives wrt mutilde of the remaining state equations which depend on attitude we substitute the approximate multiplicative correction equation (12) for R into () and () to obtain

The final 15D vehicle state F-matrix is

The complete state matrix including the static landmarks is

The states s can be referred to separately from landmarks m by writing x = [s m].  The covariance can then be written as

### Measurement and Correction

The measurement module completes the estimation loop, bringing visibility to the states both directly and indirectly through cross-covariances.  The innovations process is transformed into filtered error by application of H,

The measurement function is identical to VSLAM (section 22.9).  As in the prediction stage angle errors are multiplicative and their corresponding Jacobian blocks are computed by substituting for the true rotation matrix interms of the incremental error matrix.

W is fixed reference frame.  B is body/vehicle/IMU frame.  C is camera frame, z-optical axis.  Camera and IMU same origin.

Let Rwb = body to world be attitude, Rbc = camera to body constant, R = RwbRbc = camera to world.

Let rk be world landmarks.  mk be body landmarks, pk be camera landmarks.  hc is camera projection function, hw is world to camera coordinate transformation function

The landmark positions are initialized using a batch triangulation of the first M vehicle positions.  The solution is via the N-point perspective algorithm discussed in section (CV) which estimates 3D positions based on dead-reckoned vehicle position derived from IMU integrated attitude and velocity.

To compute H, as before substitute the first order attitude correction (Rtrue = Rhat*exp(Omega(mu))

For the kth measurement

Only postion, landmark position, and attitude have direct measurement sensitivity. Vision corrects the remaining states indirectly through their correlations with these directly influenced states.

Once a landmark feature is received sequentially compute the standard EKF equations ensuring the kth landmark appears as the kth element in Hk.

Application of the correction term deltax = Kytilde to xhat proceeds using standard addition. Since the attitude error correction for q is multiplicative, the deltamu component of deltax is transformed into the incremental quaternion qtilde as in () prior to the final update

The final complication is to account in P for the affine transformation produced by the multiplication in ().  Since deltamu is governed by (), Jr(mu) is the appropriate coordinate change matrix.  Then the mu block of P is updated as

to account for the new attitude orientation.

### VIO Simulation

The eigenvalues of the relevant estimated covariance depend on range, feature geometry, vehicle motion, IMU noise, and prior uncertainty.

### Discussion

Loop closure is not automatically built into every VIO estimator. It is inherent to a landmark-based SLAM filter like yours once the system recognizes that a newly observed feature is an already-known landmark.

Your state contains persistent world landmarks:

\[ x= \begin{bmatrix} x_{\rm IMU}\\ m_1\\ m_2\\ \vdots \end{bmatrix}. \]

When you revisit a location and observe an old landmark \(m_j\), the measurement is

\[ z_k=h(r_k,R_k,m_j)+v_k. \]

Its innovation is

\[ \nu_k=z_k-h(\hat r_k,\hat R_k,\hat m_j). \]

The Jacobian contains both current-pose and old-landmark blocks:

\[ H= \begin{bmatrix} H_r & 0 & H_\theta & 0 & 0 & \cdots H_{m_j}\cdots \end{bmatrix}. \]

The Kalman correction is

\[ \delta\hat x=PH^TS^{-1}\nu_k. \]

Because the covariance contains pose–landmark cross-correlations,

\[ P_{x_{\rm IMU},m_j}, \]

a residual against that old landmark corrects the current position, attitude, velocity, biases, and landmark estimate together. That is the loop-closing mechanism.

In your simulation, feature identity is already known: feature k always refers to landmark k. So when the vehicle later sees landmark k again, the filter automatically applies a loop-closing measurement. There is no separate special “loop closure update” needed.

The practical difficulty in real SLAM is data association:

How does the system know that this new image feature is the same physical landmark observed hundreds of frames ago?

Real systems use descriptors, image retrieval/place recognition, geometric verification, and gating to answer that question. Once the association is accepted, the EKF measurement update itself provides the loop closure.

One important limitation of your current state is that it contains only the current IMU state and landmarks—not historical vehicle poses. Therefore, a loop-closing update corrects the current pose and map, but it does not retroactively alter the saved historical trajectory samples in Xhat. A pose-graph SLAM system or a filter that retains past pose clones can visibly deform the entire stored trajectory after loop closure.

By contrast, an MSCKF discards features after using their tracks. It has no persistent landmarks to reobserve later, so it does not have intrinsic long-term loop closure; that capability must be added with a map, keyframes, or a pose-graph backend.

Think of the covariance as storing not only individual uncertainty, but also which state errors tend to occur together.

Partition the state into the IMU state \(s\) and one landmark \(m_j\):

\[ \delta x= \begin{bmatrix} \delta s\\ \delta m_j \end{bmatrix}, \qquad P= \begin{bmatrix} P_{ss} & P_{s m_j}\\ P_{m_j s} & P_{m_jm_j} \end{bmatrix}. \]

For a pixel observation of landmark \(m_j\),

\[ \nu\approx H_s\delta s+H_m\delta m_j+v. \]

The gain for the IMU-state part is

\[ K_s= \left( P_{ss}H_s^T+ P_{s m_j}H_m^T \right)S^{-1}. \]

So the IMU-state correction is

\[ \delta\hat s=K_s\nu. \]

The cross-covariance \(P_{s m_j}\) appears explicitly. It says how an error in landmark \(m_j\) is statistically related to an error in the vehicle state.

For your state ordering,

\[ H_s= \begin{bmatrix} H_r & 0 & H_\theta & 0 & 0 \end{bmatrix}. \]

For example, velocity has no direct pixel-measurement derivative, but its gain is generally nonzero:

\[ K_v= \left( P_{vr}H_r^T+ P_{v\theta}H_\theta^T+ P_{v m_j}H_m^T \right)S^{-1}. \]

Thus a pixel innovation can correct velocity even though the velocity columns of \(H\) are zero.

The same holds for gyro bias:

\[ K_{b_g}= \left( P_{b_g r}H_r^T+ P_{b_g\theta}H_\theta^T+ P_{b_g m_j}H_m^T \right)S^{-1}. \]

This is how vision estimates IMU bias: not from an instantaneous direct pixel-to-bias sensitivity, but through covariance correlations accumulated during motion.

Those correlations arise naturally:

- When a landmark is first observed, an image error can be explained partly by pose error and partly by landmark-position error. The update creates \(P_{s m_j}\neq0\).

- During propagation,

\[ P_{s m_j}^- = F_s P_{s m_j}^+. \]

The state transition links position to velocity, attitude, and biases, so correlations spread into those states.

- When the landmark is observed again—especially after returning to an old location—the new pixel residual updates every state correlated with that landmark.

Even without cross-correlation, the measurement directly corrects current position, attitude, and that landmark because both appear explicitly in \(H\). The cross-covariances are what allow it to correct velocity, biases, other correlated landmarks, and the broader accumulated drift.

Observability

The interaction between H(n) and F(n-1) allows correlations in P to build up over time.  This correlation lets future measurements diagnose past errors.  Without this accumulated correlation, measurements can only be used to correct states directly present in the measurement model, e.g. H.

### Other Aided INS

Astro inertial

GNS - can measure position directly

Radar - can directly measure 3D point (X,y,Z)

Radio

Terrain

Lidar - can directly measure 3D point (X,y,Z), lidar features are persistent returns, planes, surfaces, etc.

#### Preintegration

IMU preintegration condenses many high rate IMU outputs into a one relative motion constraint referenced to a keyframe.  Generally, IMUs provide sensor outputs at >>100HZ, whereas camera sensors are limited by exposure time to 10's of Hz.

Graph based approaches use an energy functional whose terms are comprised of all sensors combined.

#### Multistate Constraint KF

Keeps current IMUE state plus a sliding window of pas camera pose clones.  Landmarks are not accumulated.

#### Depth sensors

Typically, depth sensing technologys help most with initialization.

Stereo cameras can help with initialization, since depth (scale) are immediately known.  In practice the left and right pixels are stacked in a column vector providing dual line constraints connecting the landmark and vehicle..  The equations are solved simultaneously by the EKF thus triangulating the unknown point by intersection of the lines from two views.

## Mono SLAM

A single calibrated camera is moving with respect to a world coordinate system.  Based on detecting and monitoring 3D landmarks, estimate the camera pose and position in the world coordinate system.  The solution is an N=13 state EKF.  Quaternion is a unit quaternion with rotation q and axis qx,qy,qz.

The angular velocity is wrt the body frame. Disturbance free state model:  AWGN acceleration model

Where the pure quaternion product matrix is given in .

This doesn’t mean we are assuming constant velocity, only that any deviation from this assumption is characterized by the process noise.  This will allow the filter to accommodate the uncertainty without becoming inconsistent.  The filter has a linear state model in the spatial coordinates, linear velocity, and angular velocity described by the state transition matrix F.

The quaternion is propagated by the nonlinear state transition function f(x)

The 4 x 7 Jacobian matrix for the non linear portion of the state transition is (each non unity value should have T/2 factor)

Measurements consist of the 3D world position  of stationary landmarks detected in the image.  These landmarks are converted to body coordinates via the estimated extrinsic parameters (R(q) and r(t)) and projected onto the sensor plane using known camera intrinsic matrix .  The position of the camera (body frame B) relative to the world frame (W) is given by r(t) which is the vector pointing from the world origin to the camera center. The matrix which converts world to body is R which is a function of the estimated camera pose contained in the quaternion state vector.  The transformation from 3D to sensor frame coordinate (uk,vk,f) is given by

The measurement model  which relates states to an observation  and finally to pixel location  is

In  R is expressed in terms of its corresponding quaternion via .  These measurements are then stacked up to yield the measurement vector

The 2K x (N+3K) Jacobian matrix for this transformation is found using a symbolic tool and has nonzero entries for example looking like

The kth 1x13 row is made up of derivatives wrt uk and the camera state, and similarly for the (k+1)th row fort vk wrt the camera state.  Entries beyond 13 column are composed of the kth set of 3 derivatives wrt yk. Since the landmarks are independent, the second part of the H matrix exhibits a block structure, shifting by 2x3 matrix of zeros for each landmark.

## Stereo SLAM

## Depth SLAM

## Multistate Constraint EKF

MSCKF does not store landmark positions.  It keeps a record of M camera poses.

## Factor Graphs and Nonlinear Least Squares

Break up pdf into smaller sparse factors

energy minimization via NLLS

Describe and visualze independence

bibartite graph